BIOMEDICAL SIGNALS & APPLICATIONS

4th Year  |  Biomedical Signals & Applications

Lecture Companion — Tutorial 3

EEG Analysis & Classification  •  Theory While Scripting

| Purpose | Format | Total time |
| --- | --- | --- |
| Read this alongside the Python script — each section matches one code block | One theory section per Step; code shown; concept explained in plain language | ~60 min tutorial + ~20 min discussion |

★ Note

How to use this in class:

1. Run the code block together (instructor demo or students follow along).

2. Read the matching section in this document to understand what just happened.

3. Check the plot using the 'What to look for' checklist.

4. Use the 'Ask the class' prompts to consolidate understanding.

Steps are numbered to match the Python script exactly.

# What Are We Doing in Tutorial 3?

We have clean EEG epochs from Tutorial 2. Now we ask: what do those epochs tell us about the brain during left vs right hand motor imagery?

We approach this from four angles, each revealing something the others miss:

| Step | Analysis Method | Question it answers | Output you will see |
| --- | --- | --- | --- |
| Step 1 | Event-Related Potentials (ERPs) | Does the average brain voltage differ between conditions over time? | A waveform: voltage vs time for C3 and C4 |
| Step 2 | Power Spectral Density (PSD) | Do the two conditions produce different oscillation strengths? | A spectrum graph showing power at each frequency |
| Step 3 | Time-Frequency Analysis (TFR) | Which frequencies change, and when do those changes happen? | A 2D colour map: time × frequency, colour = power change |
| Step 4 | Feature Extraction | Can we reduce each trial to a small set of numbers that capture the difference? | A feature matrix: trials × features table |
| Step 5 | LDA Classification | Can a computer predict left vs right from those numbers? | An accuracy score and a decision boundary plot |
| Step 6 | CSP + LDA | Can we do better with an optimised spatial filter? | Higher accuracy + spatial pattern maps |

| STEP 0 | Load Preprocessed Epochs | ⏱ 3 min |
| --- | --- | --- |

## What the code does

epochs = mne.read_epochs('/tmp/motor_imagery_epochs-epo.fif')

epochs_left = epochs['left_hand'] # 0 = left cue (T1)

epochs_right = epochs['right_hand'] # 1 = right cue (T2)

## The concept: splitting by condition

The clean epoch object contains trials from both conditions (left hand and right hand imaginary movements). We split them by condition label so we can compare them directly. All subsequent analysis will be about finding what is different between these two groups.

👁 What to look for in the output:

• Left hand and right hand trial counts should be roughly equal (balanced design)

• Shape: (n_trials, 64, 561) — check that channels and time points look correct

• Time axis goes from −0.5 s (before cue) to +3.0 s (end of imagery period)

💬 Ask the class: If one condition has significantly more trials than the other (e.g. 45 left, 20 right), how might this affect a classifier trained on these trials?

| STEP 1 | Event-Related Potentials (ERPs) | ⏱ 10 min |
| --- | --- | --- |

## What the code does

evoked_left = epochs_left.average() # average all left-hand trials

evoked_right = epochs_right.average() # average all right-hand trials

evoked_left.plot() # butterfly plot: all channels

evoked_left.plot_topomap(times=[0.5, 1.0, 1.5, 2.0]) # scalp maps over time

# Compare C3 vs C4 for both conditions

## The concept: averaging cancels noise, reveals signal

Every single EEG trial is dominated by the brain's ongoing background activity — the spontaneous alpha rhythm, random fluctuations, residual noise. This 'background' is much larger than the brain's response to a specific event. We cannot see the response in any single trial.

But there is a key insight: the brain's response to a repeated stimulus is approximately the same each time (it is time-locked to the event). The background noise is random — completely unrelated to the timing of the cue. When we average many trials, the random noise cancels (positive and negative cancel each other), while the time-locked response accumulates.

🟢 Analogy: Imagine trying to see a faint pattern on a wall through a frosted glass window. On any single look, all you see is the frosted blur. But if you photograph the same spot 100 times and average the images pixel by pixel, the random blur cancels out and the underlying pattern becomes visible. ERP averaging does exactly this with EEG trials.

## The contralateral rule — why C3 and C4 matter

The motor cortex is organised contralaterally: the LEFT hemisphere controls the RIGHT hand, and the RIGHT hemisphere controls the LEFT hand. In the 10-20 electrode system:

| Electrode | Brain region underneath | Most active during... |
| --- | --- | --- |
| C3 | Left primary motor cortex | Right hand movement or imagery (contralateral) |
| Cz | Central motor cortex | Both legs / bilateral activity |
| C4 | Right primary motor cortex | Left hand movement or imagery (contralateral) |

For right-hand imagery, the LEFT hemisphere (C3) is more active. So the ERP at C3 should show a stronger or different signal for right-hand trials than for left-hand trials. The C3 vs C4 comparison plot directly reveals this lateralisation.

👁 What to look for in the output:

• Butterfly plot: many overlapping traces — look for a general divergence after the cue (t = 0)

• Topomaps at 0.5 s, 1.0 s, 1.5 s: the colour distribution shifts gradually — look for left-right asymmetry

• C3 comparison plot: the two coloured lines (left hand, right hand) should CROSS or diverge clearly somewhere after 0 s

• C4 should show the mirror image of C3 — opposite lateralisation

💬 Ask the class: In the C3 plot, which condition (left hand or right hand imagery) do you expect to produce the bigger response? Why? What does 'contralateral' mean in terms of brain anatomy?

| STEP 2 | Power Spectral Density (PSD) | ⏱ 10 min |
| --- | --- | --- |

## What the code does

# Welch's method on one epoch, channel C3:

freqs_w, psd_w = welch(epoch_data[0, idx_c3], fs=160, nperseg=320)

ax.semilogy(freqs_w, psd_w) # log scale on y-axis

# MNE method — all epochs, all channels:

psd_left = epochs_left.compute_psd(method='welch', fmin=1, fmax=45)

bp_left_alpha = band_power(psd_left, fmin=8, fmax=13) # alpha band power

## The concept: what is a power spectrum?

The EEG signal oscillates — it goes up and down. Different mental states produce oscillations at different speeds (frequencies). A power spectrum decomposes the signal and tells you: at each frequency, how much energy is present?

The Fourier transform is the mathematical tool that does this decomposition. You do not need to understand its internals — think of it as a prism that splits white light into a rainbow. The EEG signal is 'white light'; the spectrum is the 'rainbow' showing how much of each colour (frequency) is present.

| EEG Band | Frequency | What you see in this experiment |
| --- | --- | --- |
| Delta | 0.5–4 Hz | Very small — subject is awake and alert |
| Theta | 4–8 Hz | Moderate — some frontal theta may be present |
| Alpha / Mu | 8–13 Hz | THE KEY BAND — motor cortex idle rhythm; decreases during imagery |
| Beta | 13–30 Hz | ALSO KEY — active thinking and motor processing; also decreases during imagery |
| Gamma | 30–45 Hz | Very small in scalp EEG — mostly filtered or too noisy to interpret |

## Welch's method — a smarter Fourier transform

The basic FFT gives a very noisy spectrum — it fluctuates a lot. Welch's method reduces this noise by splitting the signal into overlapping windows, computing the FFT of each window separately, and averaging the results. The averaged spectrum is much smoother and more reliable.

The parameter nperseg controls the window length (here 320 samples = 2 seconds). Longer windows give better frequency resolution but average over more time. For our 3.5-second epochs, 2-second windows are a good compromise.

## Band power — one number per frequency band

The function band_power() averages the PSD over a frequency range to produce a single number per trial per channel. This is how we go from the full spectrum to a compact feature:

band_power = mean(PSD values at all frequency bins between fmin and fmax)

👁 What to look for in the output:

• The PSD plot should show the classic 1/f shape — power decreasing as frequency increases

• A bump at 8–12 Hz (the alpha/mu peak) may be visible — more prominent when the subject is relaxed

• The band power numbers printed for left vs right should be slightly different (we will exploit this in Step 4)

• If alpha power is the SAME for both conditions at C3, it suggests this subject may not show strong ERD (rare)

💬 Ask the class: The power spectrum shows more energy at low frequencies than high frequencies for all EEG. Why do you think the brain produces more low-frequency activity? Is this surprising?

| STEP 3 | Time-Frequency Analysis (TFR) | ⏱ 12 min |
| --- | --- | --- |

## What the code does

freqs_tfr = np.arange(6, 40, 1) # analyse 6 to 40 Hz

n_cycles = freqs_tfr / 2.0 # wavelet length: f/2 cycles

tfr_left = tfr_morlet(epochs_left, freqs=freqs_tfr, n_cycles=n_cycles)

tfr_left.apply_baseline((-0.5, 0), mode='percent') # express as % change

tfr_left.plot([idx_c3]) # 2D colour map at channel C3

# Difference map: right minus left

tfr_diff_data = tfr_right.data - tfr_left.data

## The concept: when does each frequency change?

A power spectrum (Step 2) tells you how much energy is present at each frequency — but averaged over the whole trial. It cannot tell you whether the alpha power was high at 0.5 seconds and low at 2 seconds, or vice versa.

A time-frequency representation (TFR) solves this. It is a 2D image: time on the horizontal axis, frequency on the vertical axis, and colour showing how much power there is at each (time, frequency) point. Think of it as a movie of the power spectrum — you can see how the spectrum evolves from moment to moment throughout the trial.

🟢 Analogy: A power spectrum is like a recipe card listing all the ingredients in a dish — flour, sugar, butter. A time-frequency map is like a video of the cooking process — you can see WHEN each ingredient was added, in what quantity, and how the mixture changed over time.

## Morlet wavelets — the tool for making TFRs

To compute the TFR, we use Morlet wavelets. A Morlet wavelet is a short oscillation at a specific frequency, tapered to zero at the edges. To find out how much 10 Hz activity is present at time t = 1.5 seconds, we 'slide' a 10 Hz wavelet along the signal and measure how well it matches the signal at each moment.

We repeat this for every frequency (6 to 40 Hz in steps of 1 Hz) and every time point. The result is the full TFR matrix.

| n_cycles parameter | Effect on the TFR | Best for |
| --- | --- | --- |
| Low (e.g. 3) | Short wavelet → good TIME precision, poor frequency precision | Detecting exactly WHEN a frequency burst occurs |
| Medium (f/2) | Balanced trade-off — the default used in this script | Most motor imagery and ERP analyses |
| High (e.g. 7) | Long wavelet → poor time precision, excellent frequency precision | Precise frequency identification; stationary signals |

## Baseline correction in TFR — percent change

We apply baseline correction to the TFR using mode='percent'. This expresses each time-frequency cell as a percentage change relative to the pre-stimulus baseline (−0.5 to 0 s). A value of −40% at (1.5 s, 10 Hz) means the 10 Hz power at 1.5 seconds is 40% LOWER than during the baseline — that is an ERD.

★ Key Concept

ERD = Event-Related Desynchronisation: power DECREASES (blue in the TFR plot)

ERS = Event-Related Synchronisation: power INCREASES (red in the TFR plot)

For motor imagery at channel C3 (right-hand imagery), expect to see:

→ Blue patch at 8–13 Hz from about 0.5 s to 3 s (mu ERD)

→ Blue patch at 18–26 Hz over the same period (beta ERD)

→ Possible red patch at 18–26 Hz after 3 s (beta rebound — motor cortex returning to idle)

👁 What to look for in the output:

• TFR left vs right side by side: the blue regions should be in DIFFERENT positions

• At C3: right-hand imagery → stronger blue (more ERD) in mu and beta bands

• At C3: left-hand imagery → less blue (less ERD) because C3 is ipsilateral to the left hand

• Difference map (right minus left): red = more power for right; blue = more power for left

• Red blob above the 0 line at 8–13 Hz after cue: this means right-hand has LESS ERD at C3 than left-hand (wait — re-read the colour scale carefully)

💬 Ask the class: The TFR shows changes in frequency content over time. What would the TFR look like for a subject who is NOT performing motor imagery — just resting quietly? What would change?

# Interlude: The Motor Cortex Story — ERD and ERS

Before we extract features, let us understand what the TFR is showing us in terms of brain physiology. This is the core scientific story of motor imagery BCI.

| Time relative to cue | What happens in the brain | What we see in the TFR |
| --- | --- | --- |
| −0.5 to 0 s (baseline) | Motor cortex is idling — neurons synchronise at 8–12 Hz (the mu rhythm) | Uniform colour across baseline — no change from baseline (by definition) |
| 0 to 0.5 s (cue appears) | Visual cortex processes the cue; attention shifts to the motor task | Very small changes; visual responses in occipital channels |
| 0.5 to 2.5 s (active imagery) | Motor cortex ACTIVATES — neurons start firing non-rhythmically and asynchronously; the mu rhythm disappears | Blue patch at 8–13 Hz (mu ERD) and 18–26 Hz (beta ERD) — power drops below baseline |
| 0 to 3 s (asymmetric) | Contralateral hemisphere is MORE active than ipsilateral | ERD is STRONGER at C3 during right imagery than during left imagery |
| 3 to 5 s (after imagery) | Motor cortex returns to its idle synchronised state — often 'overshoots' | Red patch at 18–26 Hz (beta rebound) — power rises above baseline |

🟢 Analogy: Think of the mu rhythm as a car engine idling in neutral. When the driver decides to accelerate — or even just THINKS about accelerating — the engine engages, the rhythmic idle stops, and the car does work. When the driver parks again, the engine returns to idle, sometimes briefly revving higher before settling (beta rebound). The mu ERD is the 'engaging', the beta ERS is the 'return to idle'.

★ Key Concept

Why ERD is contralateral — the single most important fact for motor imagery BCI:

Right-hand imagery → LEFT hemisphere activates → C3 shows strong ERD

Left-hand imagery → RIGHT hemisphere activates → C4 shows strong ERD

Measuring the DIFFERENCE in mu/beta power between C3 and C4 gives us

a signal that reliably distinguishes left-hand from right-hand imagery.

This difference is exactly what the classifier in Steps 4 and 5 will exploit.

| STEP 4 | Feature Extraction for Machine Learning | ⏱ 10 min |
| --- | --- | --- |

## What the code does

sel_channels = ['C3', 'Cz', 'C4', 'FC3', 'FC4']

sel_bands = {'Alpha': (8, 13), 'Beta': (13, 30)}

# For each trial: compute mean PSD in alpha and beta at each channel

X_left, feat_names = extract_band_power_features(epochs_left, sel_channels, sel_bands)

X_right, _ = extract_band_power_features(epochs_right, sel_channels, sel_bands)

X = np.vstack([X_left, X_right]) # combine: (all_trials, 10_features)

y = np.hstack([zeros(n_left), ones(n_right)]) # labels: 0=left, 1=right

## The concept: from a matrix to a vector

Each cleaned epoch is a 2D matrix: 64 channels × 561 time points = 35,904 numbers. A machine learning classifier cannot work with this raw matrix — it needs a 1D feature vector per trial: one row of meaningful numbers.

Feature extraction is the transformation from the 2D epoch to a compact 1D feature vector. Here we extract 10 numbers per trial: alpha band power and beta band power at each of 5 channels (C3, Cz, C4, FC3, FC4). These 10 numbers capture the most informative aspect of the epoch — the lateralised mu/beta ERD — while discarding the rest.

| Feature | What it measures | Why it is informative |
| --- | --- | --- |
| Alpha_C3 | 8–13 Hz power at C3 | High = left motor cortex idle (left-hand condition); Low = ERD (right-hand condition) |
| Beta_C3 | 13–30 Hz power at C3 | Same logic — beta ERD accompanies mu ERD during imagery |
| Alpha_C4 | 8–13 Hz power at C4 | Mirror of C3: High during right-hand (C4 idle); Low during left-hand (C4 ERD) |
| Beta_C4 | 13–30 Hz power at C4 | Same as Alpha_C4 but in the beta band |
| Alpha_Cz | 8–13 Hz power at Cz | Less lateralised — picks up bilateral activity |
| Alpha_FC3 | 8–13 Hz power at FC3 | Premotor area — preparation for right-hand imagery |
| Alpha_FC4 | 8–13 Hz power at FC4 | Premotor area — preparation for left-hand imagery |

## The feature matrix X

After extraction, X has shape (n_trials, 10). This is a standard 2D table — rows are trials, columns are features — exactly what scikit-learn classifiers expect. The label vector y tells the classifier which class each row belongs to (0=left, 1=right).

👁 What to look for in the output:

• Feature matrix X shape: should be approximately (80, 10) — 80 trials, 10 features

• Histogram plots: if the feature is informative, the left-hand (blue) and right-hand (red) distributions should be at least partially separated

• Features marked with an asterisk (*) in the title were statistically significant (t-test p < 0.05)

• If NO features are separated at all: check that the ICA step worked and that the TFR showed actual ERD

💬 Ask the class: We chose only 5 channels and 2 frequency bands, giving 10 features. What would happen if we used all 64 channels and 5 bands (320 features) with only 80 training trials? This is called the 'curse of dimensionality'.

| STEP 5 | LDA Classification | ⏱ 10 min |
| --- | --- | --- |

## What the code does

X_log = np.log10(X + 1e-30) # log-transform for Gaussian distribution

pipeline = Pipeline([

('scaler', StandardScaler()), # zero-mean, unit-variance scaling

('lda', LinearDiscriminantAnalysis()),

])

cv = StratifiedKFold(n_splits=5, shuffle=True)

scores = cross_val_score(pipeline, X_log, y, cv=cv)

print(f'Mean accuracy: {scores.mean():.2%}')

## The concept: a classifier finds the dividing line

LDA (Linear Discriminant Analysis) is the workhorse classifier for motor imagery BCI. It works by finding the single direction in feature space that best separates the two classes. Geometrically: plot all left-hand trials as blue dots and all right-hand trials as red dots. LDA finds the straight line that divides the blue cloud from the red cloud most cleanly.

The 'best' dividing line is the one that maximises the separation between the two group centres (so the classes are far apart) while minimising the spread within each group (so each class is tightly clustered). LDA finds this line analytically in one step — no iterative training needed.

🟢 Analogy: Imagine two overlapping piles of gold coins and silver coins on a table. LDA finds the line that divides the table so that the gold pile is mostly on one side and the silver pile on the other. Anything new that falls to the left of the line: predict gold. Anything to the right: predict silver. The line is chosen to make as few mistakes as possible.

## Why log-transform before LDA?

Band power values have a heavily skewed distribution — most trials have low power, but a few have very high power (due to residual artefacts or spontaneous alpha bursts). This skewed distribution violates LDA's assumption that each class follows a Gaussian distribution. Taking the log10 compresses the large values and makes the distribution approximately symmetric and Gaussian, which makes LDA work better.

## Cross-validation — honest accuracy estimation

Never test a classifier on the same data it was trained on — it will appear perfect because it memorised the training examples. Instead, we use 5-fold cross-validation:

- Split all 80 trials into 5 equal groups (folds) of 16 trials each

- Round 1: train on folds 2,3,4,5 (64 trials) → test on fold 1 (16 trials)

- Round 2: train on folds 1,3,4,5 → test on fold 2

- Repeat for all 5 folds — every trial is used as a test exactly once

- Final accuracy = average of the 5 test accuracies

| Accuracy range | Interpretation |
| --- | --- |
| ~50% | Chance level — the classifier is not learning anything useful |
| 55–65% | Weak — some signal present but features or classifier need improvement |
| 65–75% | Typical for naive band power + LDA on motor imagery |
| 75–85% | Good — well-tuned pipeline, subject shows clear ERD |
| 85%+ | Excellent — very clean data or advanced methods (CSP, deep learning) |

👁 What to look for in the output:

• Per-fold accuracies: some variance is normal — fold-to-fold variation of ±5–10% is expected

• Mean accuracy should be above 50% — if not, the features may not be informative enough

• Decision boundary plot: the two coloured clouds of dots should be somewhat separated

• The decision boundary (coloured background) should broadly match where the dots are

💬 Ask the class: Our classifier uses 5-fold CV and gets 65% accuracy. A student argues 'that is only a little above 50%, so it is basically chance'. How would you explain the statistical significance of 65% with 80 trials? (Hint: think about how many trials would need to be wrong for chance.)

| STEP 6 | CSP + LDA — The Motor Imagery Gold Standard | ⏱ 12 min |
| --- | --- | --- |

## What the code does

from mne.decoding import CSP

csp = CSP(n_components=4, log=True)

pipeline_csp = Pipeline([('csp', csp), ('lda', LinearDiscriminantAnalysis())])

X_raw = np.vstack([epochs_left.get_data(), epochs_right.get_data()])

scores_csp = cross_val_score(pipeline_csp, X_raw, y, cv=cv)

csp.plot_patterns(epochs.info) # show spatial patterns on scalp

## The concept: let the data choose the best channels

In Step 4, we chose which channels to use (C3, Cz, C4, FC3, FC4) based on our neuroscientific knowledge. But what if the subject's motor cortex is slightly shifted compared to the standard electrode positions? What if a weighted combination of 10 channels gives a stronger signal than any single channel?

CSP (Common Spatial Patterns) answers these questions automatically. It finds the optimal linear combination (weighted sum) of all 64 channels that maximises the power difference between the two conditions. Instead of us choosing channels by hand, CSP learns from the data which combination works best.

## What CSP actually computes

CSP finds spatial filters — each filter is a recipe saying 'multiply channel 1 by this weight, channel 2 by this weight, ... channel 64 by this weight, and sum them all up'. The result is a 'virtual channel' that is specifically designed to maximise left-right discriminability.

| CSP Component | What it maximises | Expected scalp pattern |
| --- | --- | --- |
| Filter 1 | Power for left-hand condition | Focal activation near C4 (right hemisphere) |
| Filter 2 | Power for left-hand condition | Second-best left-hand discriminating pattern |
| Filter 3,4 | Intermediate | Less interpretable — not very discriminative |
| Filter 5,6 | Power for right-hand condition | Second-best right-hand pattern |
| Filter 7,8 | Power for right-hand condition | Focal activation near C3 (left hemisphere) |

## Log-variance features from CSP

For each CSP component and each trial, we compute the log of the variance of the filtered signal. The variance measures how much the signal fluctuates — equivalent to the power. A trial with strong ERD in a particular component will have lower variance. These log-variance values are then fed to LDA as features.

★ Warning

Important: plot the spatial PATTERNS, not the spatial FILTERS.

csp.plot_patterns() shows the correct thing — this is what MNE does by default.

The patterns show where each component's signal comes from on the scalp.

The filters (the mixing weights) look different and are harder to interpret neurologically.

👁 What to look for in the output:

• CSP accuracy should be HIGHER than the band power LDA accuracy — typically 5–15% better

• Pattern 1 and Pattern 2 topo maps: should show focal activity near C4 (right hemisphere)

• Pattern 3 and Pattern 4: should show focal activity near C3 (left hemisphere)

• If the CSP patterns look random with no clear spatial structure: the data may not have enough trials or the ERD is too weak

💬 Ask the class: CSP learns the spatial filter from the training data. What is the danger of using CSP if you have a very small dataset (e.g. only 20 trials per class)?

# Summary — The Complete Analysis Pipeline

Every concept and MNE command from Tutorial 3 in one place:

| Step | MNE / Python command | What it produces |
| --- | --- | --- |
| 0 — Load | mne.read_epochs(); epochs['left_hand'] | Clean epochs split by condition |
| 1 — ERP | epochs.average(); evoked.plot(); plot_topomap() | Average waveform + scalp maps over time |
| 2 — PSD | compute_psd(method='welch'); band_power() | Spectrum + one power value per (channel, band) |
| 3 — TFR | tfr_morlet(); apply_baseline(mode='percent') | Time × frequency colour map showing ERD/ERS |
| 4 — Features | extract_band_power_features() → X matrix | Feature table: n_trials × 10 numbers |
| 5 — LDA | Pipeline([StandardScaler, LDA]); cross_val_score() | Accuracy ± std; decision boundary plot |
| 6 — CSP + LDA | CSP(n_components=4); Pipeline([CSP, LDA]) | Better accuracy + interpretable spatial patterns |

★ Key Concept

The complete motor imagery BCI pipeline — connecting everything:

Clean EEG (Tutorial 2)

→ Visualise ERD at C3/C4 (ERP + TFR)

→ Extract band power features from mu and beta bands

→ Log-transform for Gaussian distribution

→ Train LDA with 5-fold cross-validation

→ Apply CSP for data-driven spatial filtering

→ Predict left vs right hand imagery from new trials

This pipeline is the foundation of commercial EEG-based BCI devices

used clinically for motor rehabilitation and assistive communication.

| Final accuracy comparison | Why? |
| --- | --- |
| Chance (random guess) | 50% | Two classes — guessing randomly is right half the time |
| Band power LDA | ~60–70% | Simple but informed features; works with small datasets |
| CSP + LDA | ~70–80% | Data-driven spatial filter finds the optimal channel combination |
| Deep learning (EEGNet) | ~75–85% | Learns spatial and temporal patterns jointly; needs more data |